Quantitation of human enteric viruses as alternative indicators of fecal pollution to evaluate wastewater treatment processes

Audrey Garcia, Tri Le, Paul Jankowski and Miguel Uyaguari-Díaz*

Department of Microbiology, Faculty of Science, University of Manitoba, Winnipeg, MB, Canada R3T 2N2

* Corresponding Author:

Miguel Uyaguari-Díaz

45 Chancellors Cir, Winnipeg, Manitoba, R3T 2N2, Canada

Email address: Miguel.Uyaguari@umanitoba.ca

ABSTRACT

We investigated the potential use and quantitation of human enteric viruses in municipal wastewater samples of Winnipeg (Manitoba, Canada) as alternative indicators of contamination and evaluated the processing stages of the wastewater treatment plant. During the fall 2019 and winter 2020 seasons, samples of raw sewage, activated sludge, effluents, and biosolids (sludge cake) from the North End Sewage Treatment Plant (NESTP), the largest wastewater treatment plant in the City of Winnipeg, were collected. DNA and RNA enteric viruses, as well as the uidA gene found in Escherichia coli (E. coli) were targeted in the samples collected from the NESTP. Total nucleic acids from each wastewater treatment sample were extracted using a commercial spin-column kit. Enteric viruses were quantitated in the extracted samples via quantitative PCR using TaqMan assays.

The average gene copies assessed in the raw sewage were lower but not significantly different (p-values ranged between 0.0547 and 0.7986) than the average gene copies assessed in the effluents for Adenovirus and CrAssphage (DNA viruses), Pepper Mild Mottle Virus (RNA virus), and uidA in terms of both volume and biomass. A significant reduction of these enteric viruses was observed consistently in activated sludge samples compared with those for raw sewage. Corresponding reductions in gene copies per volume and gene copies per biomass were also seen for uidA but were not statistically significant (p-value = 0.8769 and p-value = 0.6353, respectively). The higher gene copy numbers of enteric viruses and E. coli observed in the effluents may be associated with the 12-hour hydraulic retention time in the facility. Enteric viruses were found in gene copy numbers at least one order of magnitude higher than the E. coli marker uidA. This may indicate these enteric viruses can survive the wastewater treatment process and therefore suggest that viral-like particles are being released into the aquatic environment. From our results, we propose the use of Adenovirus, CrAssphage, and Pepper mild mottle virus as viral indicators of fecal pollution.

INTRODUCTION

The human fecal waste present in raw sewage (RS) contains pathogens that can cause numerous diseases. This can have a huge negative impact to public and aquatic health and the economy (Stachler et al., 2017). Wastewater treatment plants (WWTPs) serve as protective barriers between communities and the environment by reducing the organic matter present in wastewater. Water quality is currently assessed using traditional markers such as coliforms and Escherichia coli, leaving other microbes such as viruses largely unexplored. The North End Sewage Treatment Plant (NESTP) in Winnipeg, Manitoba handles 70% of the city’s wastewater treatment, serving over 400,000 people (City of Winnipeg, 2019a). The treatment process at the NESTP first involves RS undergoing primary treatment during which solids are removed. It then gets processed to the stage of activated sludge (AS), in which a heterotrophic cocktail of bacteria and protozoa degrades organic matter present in solid waste. After this treatment cycle, AS is removed. The treated water is disinfected and is discharged as effluents (EF) into the river (City of Winnipeg, 2019a). Approximately 200 million liters of EF are discharged per day (City of Winnipeg, 2019b).

The main indicator of contamination used in wastewater treatment screening is E. coli, a fecal coliform bacterium (Hood et al., 1982 – [aem00170-0140.pdf (nih.gov)]). It is present in the gut of humans and warm-blooded animals and widely used as the main indicator of fecal pollution including during the wastewater treatment process. Nevertheless, the use of only fecal bacteria indicators excludes other possible pathogen groups present, such as human enteric viruses. Targeting these viruses in EF could be an alternative method to monitor the wastewater treatment process. Within this context, CrAssphage (a DNA enteric virus) and the Pepper Mild Mottle Virus (PMMV) have been found to be a dominant presence in wastewater samples. in a study conducted by Dutilh et al. (2014), the DNA CrAssphage genome was targeted in a human fecal sample. With further bioinformatics testing, it was predicted that the CrAssphage genome is highly abundant, and it was identified in 73% of human fecal metagenomes surveyed (Dutilh et al. 2014). In a study conducted by Zhang et al. (2006), the most abundant fecal virus they found in dry weight fecal matter was the plant RNA virus, Pepper mild mottle virus (PMMV).

In the present study, samples of RS, AS, EF, and biosolids/sludge cake (SC) from the NESTP were collected (during fall 2019 and winter 2020) to investigate the potential of quantitating human enteric viruses in wastewater samples as complementary indicators of contamination to evaluate the processing stages of wastewater treatment. DNA enteric viruses in this study include Adenovirus (AdV) and CrAssphage, while RNA enteric viruses include PMMV, Noroviruses (NoV) of the genogroups GI & GII, Astrovirus (AstV), Sapovirus (SaV) and Rotavirus (RoV). We also studied the presence of a molecular marker for E. coli, the uidA gene, in the samples collected from the NESTP. An overview of the workflow is illustrated in Figure 1.

MATERIALS AND METHODS

Sample Collection

A liter of RS, AS, EF, and 1 kg of SC were collected from the NESTP during fall 2019 and winter 2020. Each sample was sealed in a 1L sterile container lined with a sterile plastic bag. Samples were collected on October 22nd, 2019 (Event 1) and November 28th, 2019 (Event 2) in the fall season. In the winter season, samples were collected on December 18th, 2019 (Event 3) and February 6th, 2020 (Event 4). SC samples were collected on the days of Events 3 and 4. A liter of raw sewage, activated sludge, effluents, and one kilogram of sludge cake were collected and sealed in a one-liter sterile container lined with a sterile plastic bag for each event. These samples were kept at 4°C and processed within 24 h of collection.

Assessment of Ultrafiltration for Viral Recovery Efficiency

Armored RNA (Asuragen, Inc., Austin, TX, USA), an artificial virus, was used to assess recovery efficiency of the ultrafiltration method here employed. We spiked in 40,000 copies of Armored RNA into 120 ml raw sewage samples collected in duplicates from the NESTP, but not part of this study. Primers (381F: 5’- AGCCTGTCAATACCTGCACC-3’ and 475R: 5’- CACGCTTAGATCTCCGTGCT-3’), and probe (420P: 5’ Cy5-AGAGTATGAGAGGTCGACGA-TAO 3’) were designed using Primer design tool of Geneious Prime version 2021.1.1 (https://www.geneious.com) and targeted a 95-bp region within the Armored RNA genome. This targeted 95-bp fragment was sent to Integrated DNA Technologies (IDT, Inc., Coralville, Iowa) to synthetize a gBlock construct. Serial dilutions of this synthetic fragment were used to generate standards and quantify GCNs of Armored RNA via reverse transcription quantitative PCR (RT-qPCR). Standards and raw sewage samples were run in triplicates.

Thermal cycling reactions were performed at 50 °C for 5 minutes, followed by 45 cycles of 95 °C for 10 and 60 °C for 30 seconds on a QuantStudio 5 Real-Time PCR System (Life Technologies, Carlsbad, CA, USA).

Each 10-μl RT-qPCR mixture consists of 10 μl of 2.5 µL of 4X TaqMan Fast Virus 1-Step Master Mix (Life Technologies, Carlsbad, CA, USA), 400 nM each primer, 200 nM probe, 2.5 μl of template and ultrapure DNAse/RNAse free distilled water (Promega Corporation, Fitchburg, WI, USA).

Ultrafiltration of Wastewater Samples

Each wastewater treatment sample (RS, AS, and EF), including Millipore Milli-Q water as a negative control, was first filtered via a funnel and cheesecloth to remove any solid waste or debris. 140 mL of each wastewater sample was then concentrated down using an ultrafiltration method with Centricon Plus-70 filter units (Millipore Corporation, Billerica, MA). The steps of the ultrafiltration method were as follows. Using a sterile glass pipette, 70 mL of each wastewater sample was added into their correspondingly labeled sample filter cup pre-assembled with the filtrate collection cup. Each assembly was then sealed with a cap. The Centricon Plus-70 assemblies were placed into a swinging bucket rotor and centrifuged at 3000 x g for 30 minutes at 20°C. Subsequently, the filtrate was discarded, and the remaining 70 mL of the samples was added into their correspondingly labeled sample filter cup pre-assembled with the filtrate collection cup. Samples were spun down using the same speed and temperature for 45 minutes. After centrifugation, the sample filter cup was separated from the filtrate collection cup. The concentration collection cup was then turned upside down and placed on top of the sample filter cup. The device was carefully inverted and placed into the centrifuge. Centricon Plus-70 filter units were centrifuged at 800 x g for 2 minutes at 20°C. After this step, the concentrated sample was collected from the concentration cup via a micropipette. The final volume was measured for each wastewater sample. If needed, 10 mM Tris-HCl, pH 8.5 buffer (Qiagen Sciences, Maryland, MD) was added to the concentrate to make up a total volume of 250 μL. If the final volume of the concentrate was over 250 μL, Tris buffer was not added. Aliquots containing 250 μL were made and stored at 4°C and processed within 24h.

Sludge Cake Preparation for Ultrafiltration

To remove cells from the SC samples, 1X phosphate-buffered solution (PBS) and 0.01% Tween pH 7.4 were used. Approximately 30 g of SC sample per sampling event (Events 3 and 4) was collected and divided into six Falcon tubes for each event (~5-6 g per tube). Approximately 30 mL of PBS was added to each tube. The Falcon tubes filled with SC samples were homogenized by vortexing them at constant agitation for 15 minutes at 2500 rpm. These tubes were then centrifuged at a speed of 4500 x g for 50 minutes. The supernatant from each tube was subsequently recovered and transferred to a new sterile Falcon tube. For each sample event, 140 mL of supernatant was used for ultrafiltration as described previously.

Nucleic Acid (DNA/RNA) Extraction and Fluorometric Assessment

Once the final volume of concentrate was collected from each wastewater sample (as described above), we pretreated the sample with InhibitEX buffer (Qiagen Sciences, Maryland, MD) as indicated by the manufacturer. Then, QIAamp MinElute virus spin kit (Qiagen Sciences, Maryland, MD) was used to extract total nucleic acids from each wastewater sample. We followed the manufacturer’s instructions that included the use of Qiagen Protease and carrier RNA (Qiagen Sciences, Maryland, MD). Samples were eluted in 75 μL of Buffer AVE (Qiagen Sciences, Maryland, MD), quantified and stored at -80°C for downstream processes. We assessed nucleic acid concentration and purity using Qubit dsDNA high sensitivity and RNA assay kits in a Qubit 4 fluorometer (Invitrogen, Carlsbad, CA, USA). Qubit results can be found in Supplementary Materials (Table S1).

gene copies=× ×

gene copies per ml of sample=

qPCR Primers, Probes, and

gBlocks Gene Fragments

Table 1 summarizes the primers and probes used in this study. Forward and reverse primers described in Table 1 were used in the Primer-BLAST tool to extract gene target regions. Extracted regions were then uploaded to the software Geneious to verify oligonucleotide sequences associated to the flanking regions and probe. The generated sequences were sent to Integrated DNA Technologies (IDT, Inc., Coralville, Iowa) to generate the desired gBlocks constructs. IDT manufactured all the primers used for qPCR, as well as the probes Ast-P, Ring1a.2, and Ring 2.2. However, probes Sav124TP, Sav5TP, NSP3-P, AdV-P, PMMV-P, and CrAss-P were manufactured by Life Technologies (Carlsbad, CA, USA).

Quantitative PCR Assays

Taqman Environmental Master Mix 2.0 (Life Technologies, Carlsbad, CA, USA) was used for assays involving DNA enteric viruses and uidA, while 4x Taqman Fast Virus 1-Step Master Mix (Life Technologies, Carlsbad, CA, USA) was used for RNA enteric viruses. Each 10 μl qPCR reaction contained 500 nM of each forward primer and reverse primer and 250 nM of its designated probe when targeting both DNA and RNA viruses. Five μl of Environmental Master Mix was in each qPCR reaction for targeting DNA viruses, while 2.5 μl of 4x Fast Virus Master Mix was in each qPCR reaction for targeting RNA viruses. The uidA qPCR reaction consisted of 5 μl of Environmental Master Mix, 0.4 μM of each primer, 0.1 μM of probe. All qPCR reactions used 2 μl of template.

Each qPCR reaction was performed in triplicates on the ABI QuantStudio 5 PCR system (Applied Biosystems, Foster City, CA, USA). The DNA enteric viruses (AdV and CrAssphage) and uidA ran under the following conditions: 50.0°C for 2 minutes and 95.0°C for 10 minutes followed by 40 cycles of 95.0°C for 15 seconds and 60.0°C for 1 minute. The RNA enteric viruses (SaV, RoV, AstV, GI and GII NoV, PMMV) ran under the following conditions: 50.0°C for 5 minutes and 95.0°C for 20 seconds followed by 40 cycles of 95.0°C for 3 seconds and 60.0°C for 30 seconds. Raw qPCR output files can be found on GitHub [GitHub link].The volume in each well was 10 μl.

Assessment of Gene Copy Numbers by Volume and Biomass

Gene copy numbers (GCNs) were expressed in terms of sample (per mL or g of sample) and biomass (per ng of DNA or RNA). GCNs per mL of sample were calculated as previously described by Ritalahti et al. (2006). When calculating GCNs per mL of sample, the final volume recovered after filtering 140 mL of wastewater sample was used in the formula. For the SC samples, the mass of SC collected was used in the formula, giving results in units of GCNs per g of sample.

Collection of Metadata for Sampling Events

To perform Principal Component Analysis (PCA) and Spearman’s rank correlation analysis for EF samples, metadata pertinent to the sampling events was retrieved. Water quality parameters obtained from the NESTP were combined with their October 2019 monitoring data [need to make reference for this link - 2019-10 NEWPCC.xlsx (winnipeg.ca)] to complete some of the missing fields. For each value found in neither document, data interpolation was performed by taking an average of the corresponding values for the days before and after the sampling event. In addition, we searched the Government of Canada’s historical weather database to obtain the mean temperature on the sampling dates and the total precipitation over three days before each sampling event (hereafter referred to as “precipitation”) [Historical Data - Climate - Environment and Climate Change Canada (weather.gc.ca)].

The values for all parameters were transformed using log10, except for precipitation due to the presence of zero values. These variables were used with log10-transformed GCNs per mL sample for AdV, CrAssphage, PMMV, and uidA (targets with quantifiable qPCR readings for all replicates across all events) as input for downstream analyses (PCA and Spearman’s rank correlation analysis).

Data Handling, Statistical Analysis, and Data Visualization

Various applications were employed to process data at different steps of the pipeline. Input data, such as output from the qPCR instrument, was subjected to rudimentary formatting and cleaning in Microsoft Excel, which was then also used to calculate GCNs per mL or g sample and per ng nucleic acid.

R (R Core Team, 2021) and its integrated development environment RStudio (RStudio Team, 2021) were utilized to further process the data and perform statistical analyses and output visualizations. These operations included general linear models (and estimated pairwise differences) using the package sasLM version 0.6.0 [Bae, 2021], PCA (corresponding biplots were created using the package ggbiplot version 0.55 [Vu, 2011]), and Spearman’s correlation matrix using the package Hmisc version 4.5-0 [Harrell et al., 2021]. The package reshape2 version 1.4.4 [Wickham, 2007] was used to reformat these correlation matrices to make them more compatible with other data-handling tools. Other packages utilized but not mentioned here will be further detailed in Supplementary Materials (Table S2). The R script used for analysis can be found on GitHub ([GitHub link]).

Another software involved in data visualization was Tableau. Specifically, it was used to generate boxplots for GCNs per mL or g sample and per ng nucleic acid, as well as the heatmap representing the above-mentioned Spearman correlation matrix.

For all tests, a p-value of 0.05 was assumed to be the minimum level of significance.

RESULTS

For our assessment of the ultrafiltration method used in this study, the recovery efficiency of Armored RNA as measured by RT-qPCR was estimated to be between 7.14% and 8.64% for both raw sewage samples.

The GCN values for the DNA and RNA viruses and uidA were transformed into log10 form. These values then ran a general linear model Tukey-Kramer analysis, and the means of each wastewater processing stage for each target were analyzed. The GCNs were expressed in terms of volume (mL) or weight (g) of sample and biomass (ng of nucleic acids). In some cases, these values were log10-transformed for readability purposes. The presence of DNA and RNA viral gene copies and uidA in the Milli-Q water (negative control) samples across all Events 1-4 were negative. The orange-dotted lines in Figures 2-6 indicate the mean of the number of gene copies of each wastewater treatment sample across all events.

2B.

2A.

2C.

2D.

3A.

3B.

4B.

The average gene copies assessed in the RS were lower but not significantly different (p-values ranged between 0.0547 and 0.7986) than the average gene copies assessed in the EF for

the DNA enteric viruses

(AdV and CrAssphage), PMMV, and uidA (E. coli) in terms of both volume and biomass. However, the average gene copies of the DNA enteric viruses assessed in AS was significantly and consistently lower compared to RS. Corresponding reductions in gene copies per volume and gene copies per biomass were also seen for uidA, although these reductions were not statistically significant, with p-values being 0.8769 for the former and 0.6353 for the latter. However, the average gene copies assessed in the activated sludge was significantly lower compared to raw sewage and effluents. For all the aforementioned targets, there was a relatively higher number of gene copies observed in the EF across all events compared to AS samples.

NoV GI and GII were also targets for our study. Box plots of their GCNs across the different wastewater stages throughout Events 1-4 can be found in Supplementary Materials (Figure S1). In Events 1 and 2 (fall season), gene copy quantities of NoV GI were below qPCR detection limits for all samples (RS, AS, and EF). Also below detection limits were NoV GII GCNs for all samples collected in Event 2 and AS samples in Events 3 and 4 (winter season). Among the quantifiable samples, statistically significant GCN differences in terms of volume/mass and biomass were calculated for the pairs of AS-EF (p-values were 4.483 x 10-6 and 3.226 x 10-7, respectively), AS-RS (1.658 x 10-6, 1.091 x 10-5), and AS-SC (1.481 x  10-9, 4.083 x 10-7). No other significant differences were detected among treatments for CGNs of NoV GI. There was not a significant difference between the mean gene copies of NoV GII in the RS and EF samples in terms of volume (p-value = 0.7377), but the difference was significant in terms of biomass (p-value = 0.0490).  The corresponding quantities of all the other sample pairs were statistically significant when looking at both the volume/mass and biomass perspectives, with p-values ranging from 1.304 x 10-8 to 0.0046, except for AS-RS GCN difference in terms of biomass (p-value = 0.0637).

In addition, we looked at RoV gene copies across the various wastewater treatment stages from Event 1 to 4. The boxplots illustrating these results in terms of both sample and biomass can be found in the Supplementary Materials (Figure S2). RoV GCNs were below detection limit for all samples collected in Events 1 and 2. Looking at the EF-SC pair, the mean GCNs differed significantly in terms of volume/mass (p-value = 2.649 x 10-7) but not biomass (p-value = 0.4298). No other significant GCN differences could be detected between RS and AS samples in terms of both volume (p-value = 0.415509) and biomass (p-value = 0.6662). The equivalent magnitudes for the remaining pairs per volume/mass and per biomass were statistically significant, with p-values being between 7.907 x 10-10 and 0.0243.

Additionally, there was no detection of gene copies for AstV and SaV (Sav1, Sav124, and Sav5) in any of the wastewater samples across all events.

To eliminate the possibility of inhibitors or contaminants such as humic acids, additional qPCR tests using bovine serum albumin (data not shown) were conducted with environmental samples (including AS). No significant differences were observed between samples with and without the enzyme.

To investigate any potential relationship between collected data for EF samples, PCA was performed with log10-transformed variables. We found that three components (PC1, PC2, and PC3) explained 99.14% of the variance between variables. A summary of the weight of components is included in the Supplementary Materials (Table S3). PC1 and PC2 were used to create the biplot in Figure 5. Biplots for PC1 versus PC3 and PC2 versus PC3 are included in the Supplementary Materials (Figures S3 and S4).

Overall, based on the biplot of PC1 and PC2, samples from the four events were distinct from one another, as point clusters of the four events can be seen occupying different quadrants. PC1, explaining 54.9% of the observed variance, received a notable and positive contribution from COD, cBOD, BOD, and TOC. Strongly negative contributors to PC1 were mean temperature, grab filtered UVT, NOx-N, and TS. These observations were supported by subsequent Spearman’s rank correlation analysis (Figure 6), as COD, cBOD, BOD, and TOC demonstrated strongly positive correlations with one another (rho ranging between 0.800 and 0.949) (p-value < 0.005) and strongly negative correlations with mean temperature, grab filtered UVT, NOx-N, and TS (rho ranging between -0.949 and -0.800) (p-value < 0.005). PC2 explained 31.9% of the variance between sampling events and showed a strong contribution from CrAssphage, uidA, and grab temperature. This observation was also supported by the Spearman’s rank correlation analysis showing these variables having strongly positive correlation with one another (rho ranging between 0.717 and 0.922) (p-value < 0.01). Additionally, in the biplot, the axes representing E. coli and fecal coliform only pointed towards the same quadrant, something reflected in their moderately positive Spearman’s coefficient (0.632) (p-value = 0.0273). However, it is worth noting that uidA and E. coli exhibited a moderately weak negative correlation (rho = -0.307), although it was not statistically significant (p-value = 0.3313). The two parameters with the strongest contribution against PC2 were grab pH and turbidity, which strongly correlated with each other as illustrated in the Spearman’s coefficient heatmap (rho = 0.800) (p-value = 0.0018).

DISCUSSION

The ultrafiltration method used in this study was assessed and the recovery efficiency was estimated to be between 7.14% and 8.64%. This range was lower compared to other methods to concentrate viral particles such as JumboSep (13.38%± 9.11%) or skimmed milk flocculation (15.27%±3.32%) spiked-in wastewater samples and using Armored RNA as internal control (Yanaç and Uyaguari, unpublished results). Viral particles may have been sorbed to biosolids present in wastewater samples. In this context, matrix has a significant effect for recovery of viral particles. When compared to other environmental matrices such as surface water samples, recovery efficiency is higher using ultrafiltration (tangential flow filtration) (32.6%±11.81%) and skimmed milk flocculation (42.64%± 15.12%) (Francis and Uyaguari, unpublished results).

The GCNs were expressed in terms of biomass and volume (except for SC, which was expressed in gs of sample).

The higher abundance and more stable signal over time of GCNs of AdV, CrAssphage, and PMMV relative to the results for our other assays make these target more representative targets for conducting comparisons with E. coli. This persistent presence is consistent with various longitudinal studies previously performed [AdV: Seasonal and diurnal surveillance of treated and untreated wastewater for human enteric viruses - PubMed (nih.gov), Human Adenovirus Molecular Characterization in Various Water Environments and Seasonal Impacts in Riyadh, Saudi Arabia - PubMed (nih.gov), es6b01384 1..9 (nsf.gov) (also PMMV), Evaluating the fate of bacterial indicators, viral indicators, and viruses in water resource recovery facilities (nih.gov); CrAss: Critical Evaluation of CrAssphage as a Molecular Marker for Human-Derived Wastewater Contamination in the Aquatic Environment | SpringerLink (oclc.org), Applicability of crAssphage, pepper mild mottle virus, and tobacco mosaic virus as indicators of reduction of enteric viruses during wastewater treatment | Scientific Reports (oclc.org) (also PMMV), Dynamics of crAssphage as a human source tracking marker in potentially faecally polluted environments - ScienceDirect (oclc.org); PMMV: Pepper mild mottle virus in wastewater in Egypt: a potential indicator of wastewater pollution and the efficiency of the treatment process | SpringerLink (oclc.org) (also HAdV)].

A reduction of AdV, CrAssphage, PMMV and uidA GCNs was observed consistently in AS samples. This could be a result of viral particles being sorbed to larger fractions of organic matter that had been filtered by cheesecloth early in the sample-handling process. Meanwhile, the higher GCNs of viruses and E. coli observed in the EF may be associated with the hydraulic retention time (HRT) (12 hours) in the facility. In a study by ElNaker, Yousef, and Hasan published in 2018, it was shown that bacterial counts in bioreactors increased as the HRT was raised from 2 to 50 hours [Effect of hydraulic retention time on microbial community structure in wastewater treatment electro‐bioreactors - ElNaker - 2018 - MicrobiologyOpen - Wiley Online Library (oclc.org)]. Similarly, Pan et al. in 2004 concluded that the optimal HRT was between 2 and 12 hours, and such an HRT provided the hydraulic selection pressures favorable for the formation and maintenance of stable aerobic granules with good settleability and activity [The effect of hydraulic retention time on the stability of aerobically grown microbial granules - Pan - 2004 - Letters in Applied Microbiology - Wiley Online Library]. On the other hand, growth rates were found to increase as HRT was decreased from 24 to 6 hours [Characteristics_and_performance_of_aerob20160421-15571-ggj9hs.pdf (d1wqtxts1xzle7.cloudfront.net)]. As evident through the studies cited, there are varied results among investigations into the change in microbial growth rate in AS, but it can be said that at an HRT of 12 hours, there is microbial growth in AS. Therefore, it is reasonable to postulate that our target species also experienced growth during AS processing. However, for each sampling event, samples were collected within a 2-hour period from RS  AS  EF consecutively. As a result, the community quantified in EF may not completely reflect the changes occurring in the AS during sample collection. Other variables to consider are the overflow of sewage from rainy events and fluctuations in mixed liquor-suspended solids (Pérez, et al. 2019). In our study, there were 4.6 mm of precipitation for Event 1, which may have affected the results obtained. Furthermore, the viral GCNs quantified in the EF may be an overestimation of the actual number of infectious viral particles since qPCR detected both infective and non-infective agents and UV treatment influenced viral viability [Human enteric viruses in a wastewater treatment plant: evaluation of activated sludge combined with UV disinfection process reveals different removal performances for viruses with different features - Lizasoain - 2018 - Letters in Applied Microbiology - Wiley Online Library]. Thus, conservation should be practiced when interpreting these results.

Moreover, the duration of anaerobic sludge digestion is 15 days [Sewage Treatment Plants - Sewage - Water and Waste - City of Winnipeg]. In this context, GCNs of uidA in the SC were significantly reduced by anaerobic digestion. This may explain why the gene copies of uidA were lower in SC compared to all treatments. The average gene copies across all wastewater stages (RS, AS, and EF) for uidA were not significantly different in terms of both volume and biomass. When compared to uidA, enteric viruses were found to be at least one order of magnitude more abundant than the E. coli marker. This may indicate that enteric viruses are able to survive the wastewater treatment process and suggest that viral particles are being released into the aquatic environment.

Furthermore, the gene copies of CrAssphage and PMMV were highest in terms of biomass in SC. With the exceptions of CrAssphage GCN differences compared with EF (p-value = 0.30171) and RS (p-value = 0.21650), SC’s relatively greater CrAssphage and PMMV GCN was statistically significant (p-values ranged from 1.487 x 10-5 to 0.03775). Since SC is the by-product of AS and RS using anaerobic digestion, this may mean that the presence of CrAssphage and PMMV may have been lower in the wastewater being treated in the AS, but higher in the solids. However, GCNs of AdV in terms of biomass were not significantly different between the AS and SC samples. This reduction of viral gene copies of AdV may be due to efficient digestion in the AS and the anaerobic digestion process (SC). Meanwhile, plant viruses such as PMMV remain more stable (in terms of biomass) during these digestion processes (Jumat et al. 2017).

The higher presence of RoV gene copies in the EF during the winter season may indicate a higher risk of transmission during cold seasons (Atabakhsh et al. 2020), since it has been previously reported that a greater presence of RoV in EF are found during the winter season (Li et al. 2011).

The negative results of SaV (Sav1, Sav124, and Sav5) across all wastewater treatment stages during the fall and winter season are consistent with a report by Varela et al. (2018) using samples from a wastewater treatment plant in Tunisia. Their results did not support the general belief that the peak of detection of SaV occurs during the cold and rainy months of the year. However, quantitative detection of SaV in wastewater and river water in Japan showed an increased concentration of SaV in influents between winter and spring (December to May), but a decrease in SaV concentration during the summer and autumn months (July to October) (Haramoto et al. 2008). Yet another pattern of SaV presence was reported in France in 2011, as Sima et al. found the virus to be readily detected in influents but had no clear variations in numbers over the 9-month (October to June) duration of the study [Calicivirus Removal in a Membrane Bioreactor Wastewater Treatment Plant (nih.gov)]. Similarly, seasonal differences in SaV concentrations were not statistically significant in a 3-year study conducted by Song et al. in China between 2017 and 2019 [Detection of Human Sapoviruses in Sewage in China by Next Generation Sequencing | SpringerLink (oclc.org)]. As a result, there might be other factors that can influence wastewater SaV concentrations. For example, it has been speculated that pI could affect how viruses and their different strains behave in bioreactors [Takayuki_Miura_Enterovirus_norovirus_2015.pdf (sciencedirectassets.com)].

The gene copies of NoV GI and GII were below the detection limit in many of the AS samples (in terms of both volume and biomass), but still relatively abundant in the EF. This observation, similar to the aforementioned discussion regarding AdV, CrAssphage, PMMV and uidA, may be related to the HRT in the NESTP facility. A possible explanation for the greatly reduced viral GCNs in AS samples is the efficiency with which viruses are removed. Many studies have shown viral removal rates for AS treatment being higher than 95% and even as high as 99.97% [Elimination of human enteric viruses during conventional waste water treatment by activated sludge (cdnsciencepub.com); Removal of bacterial and viral indicator organisms in full-scale aerobic granular sludge and conventional activated sludge systems - ScienceDirect (oclc.org); Assessment of virus removal by a multi-stage activated sludge process - ScienceDirect (oclc.org)]. Furthermore, taking into account the observation that these viruses were found in abundance in SC samples, another contributing factor could be limitations in the sample collection process, which might not have adequately retrieved the slurry part of the sludge where the viruses may be found in greater numbers as they might have sorbed to the larger fractions of the sludge solids. The relative abundance of NoV GI and GII gene copies in RS and EF during the winter months (December and February) and the absence of NoV GII in RS in October may be due to seasonal variability (Pérez, et al. 2019). However, the presence of NoV GI and GII gene copies in RS in Event 4 (February) is consistent with a study conducted by Flannery et al. (2012), in which the concentration of NoV GI and GII gene copies in the influents of a wastewater treatment plant were significantly higher during the winter months (January to March).

In a study conducted by Morsy El-Senousy et al. (2007), high numbers of AstV gene copies (per liter) in sewage water samples (from the Greater Cairo area in Egypt) were observed at the end of autumn and during the winter months, but the AstV concentrations tended to decrease as temperatures increased. These results are significantly different from the quantitation of AstV gene copies in the wastewater samples from the NESTP. There was zero detection of AstV gene copies in all the wastewater treatment stages across all events. These results may be due to seasonal variability (Pérez, et al. 2019) and efficiency of the wastewater facility in removing this virus during the process, as discussed above.

Additionally, PCA and Spearman’s rank correlation analysis were conducted for EF samples to investigate potential connections between various physical, chemical, and biological parameters. PCA revealed that samples collected in different events from October to February were distinct from one another. This could indicate a seasonal variation in wastewater, at least in effluents. The hypothesis is consistent with previous literature [Comber et al_Seasonal variation of contaminants PEARL copy.pdf (plymouth.ac.uk)]. Organic chemical parameters such as COD, cBOD, BOD, and TOC were notable positive contributors to PC1, while mean temperature, grab filtered UVT, NOx-N, and TS most negatively contributed to PC1. These observations were validated by subsequent Spearman’s rank correlation analysis showing statistically significant coefficients. Grab filtered UVT being inversely correlated with COD, cBOD, BOD, and TOC is consistent with the widespread use of UV radiation to regulate microbial growth in a variety of medium, including water [A Critical Review on Ultraviolet Disinfection Systems against COVID-19 Outbreak: Applicability, Validation, and Safety Considerations (nih.gov)].

CONCLUSION

Our study’s primary goal is to identify human enteric viruses with the potential to become alternative indicators of fecal pollution. Towards that end, we propose AdV, CrAssphage, and PMMV as viral indicators of water quality due to their quantifiability illustrated in this investigation and past ones supporting our results, as detailed in the Discussion section. Regular monitoring of these organisms can be useful complements to current methods for assessing wastewater treatment processes. Such monitoring could be a helpful tool to assist public health efforts in the event of a viral outbreak.

Additionally, our study indicated that enteric viruses may have survived the wastewater treatment process and viral-like particles are possibly being released into the aquatic environment. Therefore, in addition to using such methods as UV radiation (which is currently used and shown in our study to be inversely correlated with biological parameters), we also suggest that WWTPs consider implementing technologies specifically designed to reduce the number of viral particles in all stages of the wastewater treatment process.

ABBREVIATIONS

AdV: Adenovirus

AS: activated sludge

AstV: Astrovirus

BOD: biochemical oxygen demand

cBOD: carbonaceous biochemical oxygen demand

COD: chemical oxygen demand

EF: effluents

GCN: gene copy number

HRT: hydraulic retention time

NESTP: North End Sewage Treatment Plant

NH4-N: ammonium-nitrogen

NoV: Norovirus

NOx-N: nitrogen oxides - nitrogen

PCA: Principal Component Analysis

PMMV: Pepper mild mottle virus

PO4-P: orthophosphate as phosphorus

RoV: Rotavirus

RS: raw sewage

RT-qPCR: reverse transcription quantitative PCR

SaV: Sapovirus

SC: sludge cake

sCOD: soluble chemical oxygen demand

TN: total nitrogen

TOC: total organic carbon

TP: total phosphorus

TS: total solids

TSS: total suspended solids

uidA: encodes the beta-glucuronidase enzyme

WWTP: wastewater treatment plant

ACKNOWLEDGEMENTS

Special thanks to Dr. Qiuyan Yuan and graduate students Palwinder Singh and Kadir Yanaç in Civil Engineering at the University of Manitoba for sample collection. Research start-up funds grant number 322388 were assigned to Dr. Miguel Uyaguari-Diaz at the University of Manitoba. We also thank Dr. Brian Mark and his staff members (University of Manitoba) for the use of laboratory equipment.

REFERENCES

- Atabakhsh, P., Kargar, M., and Doosti, A. 2020. Molecular detection and genotyping of group A rotavirus in two wastewater treatment plants, Iran. Brazilian Journal of Microbiology. 51(1):197–203. doi: 10.1007/s42770-019-00131-0.

- City of Winnipeg. 2019a. Sewage treatment plants: Introduction [online]. Available from https://www.winnipeg.ca/waterandwaste/sewage/treatmentPlant/default.stm#tab-introduction [accessed 6 April 2020].

- City of Winnipeg. 2019b. Sewage treatment plants: North End Sewage Treatment Plant (NEWPCC) [online]. Available from https://www.winnipeg.ca/waterandwaste/sewage/treatmentPlant/default.stm#tab-north-end-sewage-treatment-plant [accessed 6 April 2020].

- Dutilh, B.E., Cassman, N., McNair, K., Sanchez, S.E., Silva, G.G., Boling, L., Barr, J.J., Speth, D.R., Seguritan, V., Aziz, R.K., et al. 2014. A highly abundant bacteriophage discovered in the unknown sequences of human faecal metagenomes. Nat. Commun. 5(4498): 1−11. doi: 10.1038/ncomms5498.

- Flannery, J., Keaveney, S., Rajko-Nenow, P., O’Flaherty, V., and Dore, W. 2012. Concentration of norovirus during wastewater treatment and its impact on oyster contamination. doi: 10.1128/aem.07569-11.

- Government of Canada. 2019. Historical Data [online]. Available from https://climate.weather.gc.ca/historical_data/search_historic_data_e.html [accessed 6 April 2020].

- Haramoto, E., Katayama, H., Phanuwan, C., and Ohgaki, S. 2008. Quantitative detection of sapoviruses in wastewater and river water in Japan. Letters in Applied Microbiology, 46(3): 408-413. doi: 10.1111/j.1472-765X.2008.02330.x.

- Jumat, M.R., Hasan, N.A., Subramanian, P., Heberling, C., Colwell, R.R., and Hong, P. 2017. Membrane Biorector-Based Wastewater Treatment Plant in Saudi Arabia: Reduction of Viral Diversity, Load, and Infectious Capacity. Water 9(7): 534. doi:10.3390/w9070534.

- Li, D., Gu, A., Zeng, S.Y., Yang, W., He, M., and Shi, H.C. (2011) Monitoring and evaluation of infectious rotaviruses in various wastewater effluents and receiving waters revealed correlation and seasonal pattern of occurrences. J Appl Microbiol 110(5): 1129–1137. doi: 10.1111/j.1365-2672.2011.04954.x.

- Morsy El-Senousy, W., Guix, S., Abid, I., Pinto, R., and Bosch, A. 2007. Removal of Astrovirus from Water and Sewage Treatment Plants, Evaluated by a Competitive Reverse Transcription-PCR. Applied and Environmental Microbiology, 73(1): 164–167. doi: 10.1128/AEM.01748-06.

- Pérez, M.V., Guerrero, L.D., Orellana, E., Figuerola, E.L., and Erijman, L. 2019. Time Series Genome-Centric Analysis Unveils Bacterial Response to Operational Disturbance in Activated Sludge. MSystems, 4(4): e00169-19. doi: 10.1128/mSystems.00169-19.

- Ritalahti, K.M., Amos, B.K., Sung, Y., Wu, Q., Koenigsberg, S.S., and Löffler, F.E. 2006. Quantitative PCR Targeting 16S rRNA and Reductive Dehalogenase Genes Simultaneously Monitors Multiple Dehalococcoides Strains. Applied and Environmental Microbiology, 72(4): 2768. doi: 10.1128/AEM.72.4.2765-2774.2006.

- Stachler, E., Kelty, C., Sivaganesan, M., Li, X., Bibby, K., and Shanks, O.C. 2017.  Quantitative CrAssphage PCR assays for human fecal pollution measurement. Environ. Sci. Technol. 51(16): 9146-9154. doi: 10.1021/acs.est.7b02703.

- Varela, M., Ouardani, I., Kato, T., Kadoya, S., Aouni, M., Sano, D., and Romalde, J.L. 2018. Sapovirus in Wastewater Treatment Plants in Tunisia: Prevalence, Removal, and Genetic Characterization. Applied and Environmental Microbiology. 84(6): e02093–17–17.atom. doi: 10.1128/AEM.02093-17.

- Zhang, T., Breitbart, M., Lee, W., Run, J., Wei, C., Soh, S., et al. (2006). RNA Viral Community in Human Feces: Prevalence of Plant Pathogenic Viruses. PLoS Biology. 4(1): e3. doi: 10.1371/journal.pbio.0040003.